DHRUVA CDM

Autonomous Constellation Management & Conjunction Decision Support Platform

Hackathon Presentation — Slide Content Document

Stack: Python · FastAPI · React · Next.js · WebSocket · physics.py

Repository: github.com/jeevankumar-m/project-dhruva

Demo Video: https://youtu.be/fs9eEaVNgIA

Version 1.0.0  |  © 2026 Dhruva CDM Project

SLIDE 1

The Problem: Earth's Orbit Is Becoming a Minefield

Suggested Slide Title (on PPT):

"The Kessler Syndrome Is Not a Theory — It's a Timeline"

Key Points

Over 27,000 tracked debris objects currently orbit Earth — estimates suggest 500,000+ fragments above 1 cm, each carrying enough kinetic energy to destroy a satellite.

A single hypervelocity collision at LEO (7–8 km/s) generates thousands of new fragments, triggering a cascade — Kessler Syndrome — that can render entire orbital shells permanently unusable.

India's stakes are high: ISRO operates NavIC, Cartosat, RISAT, and emerging private constellations. Each is exposed every orbit.

Current global SSA tools — NASA CARA, ESA SST, LeoLabs — are siloed, expensive, and inaccessible to academic / startup operators.

No open-source, full-stack, physics-accurate CDM platform exists that a university lab, ISRO spinoff, or NewSpace company can deploy today.

Visual Suggestion

A timeline graphic showing debris growth from 1957 → 2024, or the Iridium-Cosmos 2009 collision visualization. Numbers should dominate the slide visually.

SLIDE 2

Introducing Dhruva CDM

"Dhruva — the North Star. The fixed point everything navigates by."

What Is It?

Dhruva CDM is a full-stack Orbital Conjunction Data Management system that autonomously detects, assesses, and responds to satellite-debris close-approach events in real time. Built from first principles — not a wrapper around someone else's propagator. Every equation is implemented, validated, and running.

Three Coordinated Capabilities

SENSE — Continuously propagate and track all objects in ECI (Earth-Centred Inertial) space.

ASSESS — Predict Time of Closest Approach (TCA) and classify risk across SAFE / WARNING / CRITICAL thresholds.

ACT — Autonomously schedule and execute avoidance maneuvers with full operational constraint validation.

Deployment

docker build -t dhruva-cdm . && docker run --rm -p 8000:8000 -p 3000:3000 dhruva-cdm

One command. Running in under 60 seconds. Open source under MIT License.

Quick Links

Demo Video: https://youtu.be/fs9eEaVNgIA

Repository: github.com/jeevankumar-m/project-dhruva

SLIDE 3

Our USP — What Makes Dhruva Different

Six Differentiators That No Existing Open Tool Combines

| Dhruva USP | Why It Matters |
| --- | --- |
| Physics-first propagation from scratch | Two-body + J2 + RK4 — not SGP4 table lookups. J2 changes RAAN by ~7°/day; ignoring it means wrong conjunction geometry hours later. |
| Dual-mode conjunction pipeline | Spatial Grid (O(1)/tick, UI-safe) + custom hand-built KD-Tree (sub-linear, stress-tested to 20,000 debris). No existing free tool does both. |
| Closed-loop autonomous avoidance | CRITICAL events auto-generate evasion + paired recovery burns. No human in the loop required. |
| Full operational constraint model | LOS blackout gating, thermal cooldown (600 s), command latency (10 s min), Tsiolkovsky fuel validation — all enforced before a burn is accepted. |
| WebSocket-streamed, sub-tick warning latency | Risk state reaches the operator display within one simulation tick — not the next poll interval. Eliminates stale-state entirely. |
| End-of-life graveyard routing | Satellites below 5% fuel are auto-flagged and routed to graveyard orbit — the IADC-compliant disposal lifecycle, fully automated. |

"ESA's SST cost €100M+ to build. Dhruva CDM is open-source, dockerised, and running in under 60 seconds."

SLIDE 4

Physics Engine — The Scientific Core

"Rigorous Astrodynamics — Every Number Has a Citation"

1. Two-Body Gravitational Acceleration

a_grav = -(μ / |r|³) · r      where μ = 398,600.4418 km³/s²

Dominant acceleration term for all Earth-orbiting objects. The baseline from which all perturbations are added.

2. J2 Oblateness Perturbation  (critically important — most student tools skip this)

s = (3/2) · J2 · μ · R_E² / |r|⁵
ax = s·x·(5z²/|r|² - 1),  ay = s·y·(5z²/|r|² - 1),  az = s·z·(5z²/|r|² - 3)

Earth's equatorial bulge causes RAAN to precess ~7°/day at ISS altitude. Without J2, conjunction predictions diverge from truth within hours. Dhruva includes the full zonal harmonic correction per Vallado (2013).

3. Fourth-Order Runge–Kutta Integration

S_(n+1) = S_n + (Δt/6)·(k1 + 2k2 + 2k3 + k4)
Local truncation error O(Δt⁵)  |  Global error O(Δt⁴)  |  Fixed step dt = 10 s

Validated against known ISS orbital period — accuracy < 0.1% error for eccentricities below 0.01.

4. Tsiolkovsky Propellant Model

m_prop = m_current · (1 − exp(−|Δv| / Isp·g₀))    Isp = 300 s,  g₀ = 9.80665 m/s²

Pre-validates every commanded maneuver against remaining fuel budget. Physically impossible burns are rejected before scheduling.

Reference Stack

Vallado, Fundamentals of Astrodynamics and Applications, 4th ed., 2013

Hoots & Roehrich, Spacetrack Report No. 3, 1980 (SGP4)

NASA CARA, NPR 8715.6A, Debris Avoidance Maneuver Planning, 2017

Alfano, Satellite Conjunction Monte Carlo Analysis, AAS 09-233, 2009

SLIDE 5

Conjunction Assessment Pipeline

"From 20,000 Debris Objects to a Ranked Risk List — in One Tick"

Naïve all-pairs assessment is O(N²). With 200 satellites × 20,000 debris = 4 million pairs per tick. Dhruva solves this with a two-stage, dual-mode pipeline.

Stage 1 — Candidate Pre-filter (two modes)

Spatial Grid (live stream mode): Debris bucketed into 500 km³ cells. Each satellite checks only adjacent cells — bounded O(1) per-tick cost regardless of debris count. Protects UI responsiveness at 50+ Hz update rate.

Custom KD-Tree (batch / stress mode): Hand-built 3D KD-tree (zero scipy dependency). Radius query per satellite — sub-linear O(log N + k) scaling. Validated at 20,000 debris objects.

Stage 2 — TCA Prediction

Both objects propagated forward over a 2-hour look-ahead horizon at 30-second RK4 samples.

Minimum separation recorded → Time of Closest Approach (TCA) and miss distance computed per pair.

Stage 3 — Risk Labelling

| Risk Level | Miss Distance | System Response |
| --- | --- | --- |
| CRITICAL | < 1.0 km | Auto-evade burn triggered immediately + recovery burn queued |
| WARNING | < 5.0 km | Operator alerted; optional manual maneuver |
| SAFE | > 5.0 km | Continue monitoring; no action required |

Output: Up to 500 ranked conjunction warnings per snapshot, sorted by miss distance and TCA.

SLIDE 6

Autonomous Maneuver Planning & Validation

"No Burn Flies Without Passing a 4-Gate Safety Gauntlet"

Validation Gate Sequence

[Command Received]
      ↓
[1] Command Latency Check    → Burn epoch must be ≥ 10 s in the future (uplink time)
      ↓
[2] Thermal Cooldown Gate    → Minimum 600 s between consecutive burns (thruster protection)
      ↓
[3] Ground Station LOS       → Satellite must be visible to a ground station at burn epoch
                               (IIT Delhi Ground Node | ISTRAC Bengaluru | Goldstone Tracking)
      ↓
[4] Fuel Feasibility         → Tsiolkovsky check: |Δv| achievable with remaining propellant
      ↓
[SCHEDULED ✓  /  REJECTED ✗  with reason code]

Autonomous CRITICAL Response

Conjunction engine detects miss distance < 1 km → threshold crossed.

Generates AUTO-EVA-* burn (velocity-direction impulse, auto-calculated magnitude).

Immediately pairs it with AUTO-REC-* recovery burn to restore original orbital plane after conjunction window passes.

Full audit trail written to burn logs — every burn timestamped, satellite identified, ΔV recorded, status marked.

Live Demo Result

15 burns executed (AUTO-EVA, AUTO-REC, operator EVASION, operator RECOVERY).

1 burn correctly rejected — EVASION-01 for SAT-Alpha-01 at 15.17 m/s — LOS constraint violated.

The rejection proves the system is safe-by-default, not just happy-path correct.

SLIDE 7

System Architecture

"Service-Oriented, Streaming, Production-Grade — From Day One"

┌──────────────────────────────────────────────────────────────┐
│           OPERATOR FRONTEND  (React / Next.js)               │
│  2D Map  │  3D Cesium  │  Bullseye  │  Analytics  │  CDM     │
└───────────────────────┬──────────────────────────────────────┘
                        │  WebSocket /orbit  (live stream)
                        │  REST /api/*       (commands)
┌───────────────────────▼──────────────────────────────────────┐
│           FASTAPI BACKEND  (server.py)                       │
│  api/routes.py  │  engine/sim_state.py   │  physics.py       │
│                 │  engine/conjunction.py  (Grid / KD-Tree)   │
└──────────────────────────────────────────────────────────────┘
      INPUT:  TLE Data  |  Cartesian Telemetry  |  CDM CSV

Key Architectural Decisions

WebSocket over REST polling: eliminates stale-state and synchronisation artifacts. Warning state reaches the UI within a single simulation tick (sub-second).

Dual execution modes: Live stream (spatial grid, UI-optimised) vs. Step/Batch (KD-tree, throughput-optimised). One backend, two operating profiles.

TLE → SGP4 → ECI pipeline: ingests real NORAD/CelesTrak catalog data; backend handles frame transformation automatically (TEME → GCRS).

Time-warp 1×–100×: compress orbital mechanics for rapid scenario rehearsal and stress testing.

Single-container Docker deploy: backend (port 8000) + frontend (port 3000) in one image. Zero-dependency setup for evaluators.

Module Responsibilities

| Module | Responsibility |
| --- | --- |
| server.py | FastAPI bootstrap, CORS, SimulationState instantiation, WebSocket /orbit endpoint |
| api/routes.py | All REST endpoints: telemetry ingest, TLE ingest, maneuver scheduling, step, snapshot, seed, time-warp, CDM CSV loader |
| engine/sim_state.py | Operational core: sim time, state vectors, maneuver queues, conjunction warnings, blackout tracking, graveyard logic, objective metrics |
| engine/conjunction.py | Conjunction pipeline: grid + KD-tree pre-filter, TCA evaluation, SAFE/WARNING/CRITICAL classification |
| physics.py | Astrodynamics: two-body + J2, RK4 integration, state propagation, Tsiolkovsky propellant model, orbital period |
| dhruva-frontend/ | React SPA: 2D map, 3D Cesium globe, Bullseye radar, analytics dashboard, maneuver workflow, burn logs |

SLIDE 8

Operator Dashboard — Orbital Insight

"Six Views. One Ground Truth. Zero Polling Lag."

| Dashboard View | What It Delivers |
| --- | --- |
| 2D Mercator Ground Track | Real-time satellite positions (cyan), debris cloud (red), 90-min historical trail + 90-min predicted trajectory (dashed), solar terminator line, ground station overlays (IIT Delhi Node, ISTRAC, Goldstone) |
| 3D Cesium Globe | Full-constellation 3D render — polar view, horizon view, flat-Earth spread — with live blackout flagging and per-satellite telemetry panel (fuel mass, altitude, longitude, drift-from-slot) |
| Conjunction Bullseye Radar | Polar radar plot per satellite: debris approach vector encoded as angle, TCA as radial distance, colour-coded risk rings (1 km / 2 km / 5 km). Expandable modal with full debris threat list. |
| Fleet Fuel & ΔV Analytics | Per-satellite propellant gauges for all 50 satellites, cumulative ΔV consumed vs. collisions avoided (rolling chart), uptime score, avoidance efficiency metric |
| Maneuver Timeline (Gantt) | Scheduled burns, executed burns (green), cooldown windows (teal), rejected commands (red) per satellite across simulation timeline — full scheduling situational awareness |
| CDM Alerts + Burn Logs | Active conjunction warnings sorted by miss distance and TCA; complete audit trail of all AUTO-EVA-*, AUTO-REC-*, and operator-initiated burns with status codes |

Graveyard Lifecycle (automated)

NOMINAL  →  GRAVEYARD CANDIDATE  →  GRAVEYARD MANEUVER SCHEDULED  →  GRAVEYARD

Trigger: fuel drops below 5% of initial propellant mass. Disposal route: above GEO (>36,000 km) for high-altitude missions; below ISS altitude (<400 km, re-entry decay) for LEO missions.

SLIDE 9

Scalability & Stress Testing

"200 Satellites. 20,000 Debris. Sub-Second Updates. Verified."

Validated Operational Limits

| Parameter | Limit / Value | Notes |
| --- | --- | --- |
| Satellites | Up to 200 | Tested at 50-satellite baseline with full conjunction pipeline |
| Debris objects | Up to 20,000 | CDM CSV loader pipeline validated end-to-end |
| Conjunction warnings/snapshot | Up to 500 | Sorted by miss distance and TCA; worst-case captured first |
| Time-warp multipliers | 1×, 2×, 10×, 25×, 100× | Scenario compression for rapid rehearsal |
| TCA look-ahead horizon | 7,200 s (2 hours) | At 30-second RK4 sample interval per object pair |
| WebSocket update rate | Sub-second per tick | Spatial grid: bounded per-tick cost regardless of debris count |
| Max burn magnitude | 15.0 m/s per burn | Enforced cap; prevents runaway fuel consumption |

Why the Custom KD-Tree Matters at Scale

Grid scan at 20,000 debris = large cell iteration count per satellite. Performance degrades linearly.

KD-tree radius query scales as O(log N + k) — dramatically faster for uniformly distributed populations.

Custom implementation (zero scipy, zero external dependency) — portable to edge compute or onboard systems.

Rebuilt each conjunction assessment call — handles dynamic debris states correctly (no stale index).

CDM CSV Stress Pipeline

Loads RTN-relative state fields from CCSDS-style CDM CSV files, anchors them to in-simulation satellite primaries, converts to ECI-like Cartesian state vectors, and injects seamlessly into the conjunction pipeline. 10,000 objects loaded, assessed, and warnings generated in a single API call.

SLIDE 10

Real-World Applicability

"Three Deployment Scenarios Where Dhruva CDM Fills a Real Gap"

Scenario 1 — ISRO / NewSpace Operators

ISRO's PSLV-C58 placed 10 satellites in a 650 km orbit — the exact LEO regime Dhruva CDM targets. A Flight Dynamics Officer using Dhruva gets real-time CDM alerts, autonomous avoidance, and a full burn-log audit without a $10M+ commercial SSA subscription. TLE ingestion from CelesTrak requires a single API call.

Scenario 2 — University Constellation Labs

IIT Delhi's Student Satellite Programme, IIST, BITS — student teams operating CubeSats have no CDM tooling available to them. Dhruva CDM deploys in a Docker container on a laptop, runs on real TLE data, and gives students the same situational awareness a Flight Dynamics Officer has — for free.

Scenario 3 — Inter-Agency Space Traffic Management

The CCSDS CDM message standard is the basis for NASA–ESA–JAXA data exchange. Dhruva CDM's architecture is explicitly designed for extension to CCSDS format (roadmap), enabling it to become a node in a federated, multi-agency STM network.

Alignment with National Priorities

India Space Policy 2023 mandates national SSA capability development — Dhruva CDM is a direct contribution.

ISRO's SSA Control Centre (Bengaluru) — Dhruva CDM's architecture maps directly to its operational mission.

IN-SPACe framework encourages open-source tooling for commercial space operators — Dhruva CDM is MIT-licensed.

IIT Delhi Ground Station is pre-configured as a named ground station node in Dhruva CDM.

SLIDE 11

Live Demo Highlights & Key Results

"The Numbers from a Live Simulation Run"

Simulation Parameters

50 active satellites  |  20 seeded debris  +  10,000 CDM CSV debris loaded (20,020 total tracked objects)

Time-warp: variable (1× – 100×)  |  Look-ahead horizon: 2 hours  |  RK4 sample: 30 s

Mission Metrics (from live run)

| Metric | Result |
| --- | --- |
| Total ΔV consumed (fleet) | 124.11 m/s |
| Fleet uptime | 92.74% |
| Uptime score | 80.42 |
| Outage satellite-seconds | 18,206 s |
| Total burns logged | 16 (15 executed / 1 correctly rejected) |
| Active CDM warnings (live sample) | 3 SAFE — 7.636 km (TCA 1080 s), 14.582 km (TCA 1800 s), 36.157 km (TCA 60 s) |
| Graveyard satellites | 0 — propellant budgeting maintained entire 50-satellite fleet above EOL threshold |
| Max simultaneous blackouts | 12 satellites (tracked and surfaced to operator simultaneously) |

The Genesis Story (Origin Proof-of-Concept)

Stage 1: 2D simulation — predicted collision point identified on look-ahead horizon. Satellite (green) and debris (red) on separate tracks.

Stage 2: Orbits converged, predicted collision point migrated toward satellite position — threshold crossed — evasion burn scheduled.

Stage 3: Post-burn trajectory confirmed diverged from debris track — closed-loop autonomy validated in the very first prototype.

The Rejected Burn — Why It Matters

EVASION-01 for SAT-Alpha-01 (15.17 m/s) was rejected by the LOS/cooldown validation gate. This is not a failure — it is proof that the system is safe-by-default. A burn that would violate an operational constraint is rejected with a reason code, not silently ignored. That is production-grade behaviour.

SLIDE 12

Roadmap & Vision

"Dhruva CDM v2.0 — The Path to Operational Readiness"

Immediate Extensions (3–6 months)

Live NORAD TLE ingestion (CelesTrak / Space-Track.org) — automated SGP4 seeding from real orbital catalog; real objects, real risk assessments.

Probabilistic collision avoidance (Pc) — replace threshold-based labelling with full covariance Foster/Alfano 2D projection method, matching the NASA CARA Pc > 10⁻⁴ operational threshold standard.

Medium-Term Research Track (6–18 months)

Higher-fidelity propagation — J3–J6 zonal harmonics, NRLMSISE-00 atmospheric drag, solar radiation pressure → extends accurate propagation from hours to days; critical for VLEO objects.

Reinforcement Learning maneuver planner — policy trained on simulated conjunction scenarios, minimising fleet-wide ΔV while maximising uptime and respecting thermal cooldown constraints (Gao et al., arXiv:2009.04050).

Long-Term Vision

CCSDS CDM standard API — interoperability with NASA, ESA, JAXA, ISRO data exchange frameworks → Dhruva as a node in federated Space Traffic Management.

Onboard autonomy migration — evasion planner packaged as a radiation-tolerant flight software module with deterministic WCET guarantees → fully autonomous onboard avoidance for satellites in prolonged blackout.

The star Dhruva does not move. Everything else navigates around it.

Our goal: build the fixed reference point for India's space safety infrastructure.

PRESENTER DELIVERY NOTES

Slide 1 — Open Hard

Quote the fragment count (500,000+) and let it land before moving on. Do not rush past it. Let the judges absorb the scale of the problem before you offer the solution.

Slide 3 — Say the Comparison Line Out Loud

"ESA's SST cost over a hundred million euros. This runs in 60 seconds on a laptop." Pause after saying it. It will land.

Slide 4 — On Physics Depth

You don't need to derive the math. Just say: "We implemented this from scratch — not a library call. Every equation is in physics.py and cites Vallado 2013." That alone impresses an ISRO judge.

Slide 5 — The O(N²) Line

Say the number: 4 million pairs per tick. Then explain how the pipeline reduces it. Concrete numbers always beat abstract complexity notation for a general audience.

Slide 6 — Walk the Gauntlet Like a Checklist

Read through the 4 gates slowly — Latency, Cooldown, LOS, Fuel. Let it sound procedurally rigorous. Judges from ISRO / DRDO will recognise this as flight-software thinking.

Slide 11 — Lead with the Rejected Burn

Open Slide 11 with: 'The most important result isn't a burn that executed — it's the one we correctly rejected.' This flips the frame from feature showcase to safety assurance.

Slide 12 — Close with the Tagline

The Sanskrit meaning of Dhruva landing at the very end of the presentation is a strong, memorable close. Deliver it slowly. Don't rush the Thank You slide after it.